# Capacitance Based Neuromorphic Computation Survey and Simulation

# Abstract

# Introduction

Neuromorphic computation is the study of designing and building computers that mimic the behavior of the human brain. In recent years, advancements in the field have led to novel research utilizing devices known as memcapacitors[1]. These devices are capacitors arranged to allow the system to have memory based on previous inputs or weights. By measuring the weights between nodes, the system intrinsically mirrors the behavior of neural nets.

Until early 2025, the devices were theoretical and simulation-based, when a team in Japan built the first memcapacitor circuit. The circuit was tested by classifying digits from the MNIST dataset accurately identifying 94% of the images and achieving an energy efficiency of 163 GOPS/W.[2] As the demands for high performance computation rise, the demand in energy for these systems rises proportionally.[1] This has led to the exploration of new ideas that allow for computation to have the same, if not better, levels of performance while reducing energy costs.

This project explored the simulation of memcapacitors both in their computational ability and their energy efficiency. LTSPICE simulations were used to model the behavior of the circuit in R. Oshio, T. Kuwahara, et al.[2] The intent of this project is to determine if the circuit can be digitally reproduced and simulated by utilizing a lower complexity program, LTSPICE, and similar python programming. Energy efficiency and prediction accuracy were utilized to determine if this intent was realized.

# Related Works

# Methodology

This project reproduces and evaluates the neuromorphic architecture described by Oshio et al. (2025), which implements multiply–accumulate (MAC) operations using binary-weighted capacitor synapses.[2] Our methodology consists of two major components:

- Python-based neural weight generation and preprocessing, and

- LTspice circuit-level replication of a single capacitive synapse, followed by timing analysis.

Before designing the model, the neuromorphic system proposed by Oshio et al. was evaluated to determine which elements could be feasibly replicated. In the original work, each synapse contains four capacitors with binary-weighted capacitances (20 fF, 40 fF, 80 fF, 160 fF) forming a 4-bit magnitude plus sign bit. These capacitors are selectively connected to an intermediate node (y⁺ or y⁻), precharged to VDD, and then discharged through a transistor chain (Td) during computation. The discharge time until an inverter flips serves as the encoded MAC output (the "inversion interval") .

Full replication was outside project scope, the team modeled a single synaptic cell including: the four binary-weighted capacitors, NMOS pass transistors for weight selection, a discharge transistor driven by a pulse input, and a CMOS inverter pair to detect the discharge threshold.

To provide realistic synaptic weights for the circuit, a Python program was developed in VS Code. The program performs the following:

- Loads the MNIST dataset using PyTorch.

- Trains a single-layer network (28×28 → 16 nodes) to mirror the 16-weight input structure used in the replicated circuit.

- Extracts the trained weight vector and normalizes values to a 0–1 range.

- Scales the weights into analog voltages (0–1.8 V) corresponding to the input coding used in the neuromorphic circuit.

- Outputs the 16 voltage values into a CSV file.

The neuromorphic synapse model was implemented in LTspice using the following components.

Four ideal capacitors were created with the same capacitance values (20 fF, 40 fF, 80 fF, 160 fF) and gated individually using NMOS switches. These switches correspond to the w₀–w₃ bits of the 4-bit synapse weight.

The original chip uses a global charging signal c to charge yⱼ to VDD before computation.
LTspice replicates this using an explicit [.ic V(y)=1.8] directive, ensuring the intermediate node begins fully charged.

A single NMOS device, equivalent to the Td stack in the hardware, receives a short-duration PULSE input simulating the discharge signal, d. Activating this path causes the capacitors to discharge through the transistor, with heavier (larger capacitance) weights producing slower voltage decay.

A CMOS inverter pair was added to convert the analog discharge into a digital-level signal z. As in the original design, the output transition occurs when node y crosses the inverter threshold (~0.9 V). A .meas statement captures the exact switching time.

For each of the four possible weight combinations (1-bit, 2-bit, 3-bit, and 4-bit enabled), the following simulation protocol was used:

- Load the selected combination of capacitors via MOSFET gate voltage (0 V = disconnected, 1.8 V = connected).

- Apply initial conditions to precharge node y.

- Run a 20 ns transient simulation with a 20 ps timestep.

- Record V(y) and V(z) over time.

- Measure inverter delay, defined as the time when V(z) crosses 0.9 V for the first rising edge.

The expected linear relationship between total capacitance ratio and inverter delay was used as the baseline for evaluating simulation correctness.

# Experimental Results

# Conclusions/Future work

# References

[1] Y. Ishisaki et al., “Analog Memcapacitor by Ferroelectric Capacitor and Its Application to Spiking Neuromorphic System,” IEEE Transactions on Electron Devices, vol. 71, no. 8, pp. 4626–4630, Aug. 2024, doi: 10.1109/TED.2024.3408783.

[2] Y. Lecun, L. Bottou, Y. Bengio, and P. Haffner, “Gradient-based learning applied to document recognition,” Proceedings of the IEEE, vol. 86, no. 11, pp. 2278–2324, Nov. 1998, doi: 10.1109/5.726791.

[3] R. Oshio, T. Kuwahara, T. Aoki, M. Kimura, and Y. Nakashima, “Neuromorphic system using capacitor synapses,” Sci Rep, vol. 15, no. 1, p. 3954, Jan. 2025, doi: 10.1038/s41598-025-87924-6.

# Contributors

Trenton Bailey  - tmb0013@arizona.edu

Zoe Diehl - zdiehl@arizona.edu

David Sanders Jr., DVM  - dlsanders@arizona.edu